Work Instructions: Replacing the MCP and SIC Fan Filters

Parts Needed:

- Fan Filter Replacement (CCS-3238200)

Tools Needed:

- High-Pressure Air

- Dawn Dish Soap

Procedure:

- To remove the filter, flip the blue clip upwards and the cover opens, revealing the fan filter.

- Clean the fan filters using high-pressure air or a vacuum to remove excess debris.

- If the filters are not clean enough, you can use dawn dish soap and water. The filters must be 100% dry before reinstalling.

- If you get the filters wet, make sure to install spare filters while the original filters dry.

- To reinstall, simply place the filter back in the port, close the cover, then snap the blue clip to secure the cover.

Instructional Video:

MCC - SIC Fan Filter on Vimeo