Work Instructions: How to Replace an Air Cylinder

Parts Needed

- Tire Shine Air Cylinder (P-TB-205)

- Wrap Retract Air Cylinder (P-M2035)

- Blast Gate Air Cylinder (P-C2008)

- All-In-One Electric Combo Unit Air Cylinder (P-M2213)

Tools Needed

- Pliers

- Hammer

Procedure

- Push in a wash stop

- Turn off the air going to the air cylinder in the POD.

- Disconnect both airlines from the air cylinder.

    - Record what airlines go into what port.

- Using a pair of pliers, remove the cotter pins holding the bolts at both ends of the air cylinder.

- Remove the bolts holding the air cylinder in place

    - A hammer may be necessary to help remove the bolts.

- Replace the old air cylinder with a new one.

- Secure the air cylinder with the bolts and cotter pins.

- Reconnect both airlines at the correct ports.

- Turn the air going to the POD back on.

- Pull out the wash stop.

- Verify that the new air cylinder is working

Instructional Video

Replacing an Air Cylinder on Vimeo