### Work Instructions: Bay Wash Cat Pump Oil Change

Parts Needed:

- Cat Pump Oil (P-SE-PG-220) https://shop.tommycarwash.com/Cat-Pump-Oil-6100

Tools Needed:

- Safety glasses

- Adjustable wrench

- Oil Drain Pan

- Funnel (if wanted)

Procedure:

1. Turn the power off to the Bay wash pump. Place a Lock-out Tag-out on the disconnect.

2. Turn the bay wash pump on Tommy Controller to “OFF”.

3. Line up the oil drain pan with the oil drain shaft to the bay wash pump.

4. Take the plug off of the ball valve on the oil drain on the bay wash pump.

5. Turn the ball valve to the open position so oil flows out of the drain.

6. Take the red cap off of the top of the bay wash pump to the oil fill port.

7. Once the oil is drained completely, put the plug back into the ball valve and turn the ball valve back to the closed position.

8. Put a funnel in the oil fill port to fill the bay wash pump with oil until the oil level reaches just over the middle of the sight glass.

9. Once full put the red cap back on to the oil fill port. Only hand tighten it as It is a plastic cap and can break if overtightened.

10. Remove Lock-out Tag-out from Bay was pump disconnect in M.C.C.

11. Turn the Bay wash pump back to “Auto” on Tommy Controller.

12. Recycle the old oil.

Instructional Video:

### Cat Pump Oil Change on Vimeo