# Work Instructions: High-Pressure Pump Oil Change

Preliminary Note:

- Check oil levels monthly, and replace if needed

Parts Needed:

- 7 Quarts of Hydra-Cell 10W30 Oil (P-SE-2122): P-SE-2122|Tommy Car Wash

Tools Needed:

- Safety glasses and gloves

- Adjustable wrench

- Pipe wrench/Vise grips

- Oil drain pan or bucket

- Flexible funnel with hose or tube (optional)

Procedure:

- Turn off HPP1 and/or HPP2 functions on iPad

- Turn off the disconnects for HPP1 and/or HPP2 in the MCC.

- Place a lockout tag out on the disconnects.

- Loosen and remove the red breather cap on top of the pump.

- Set a drain pan near the oil drain shaft.

- Remove the drain plug from the oil drain shaft. If the shaft spins, use a pipe wrench or vise grip to hold it in place. If you have a ball valve setup, remove any plugs and open the ball valve.

- Allow all of the oil to drain into the pan, and reinstall the drain plug or close the ball valve.

- Remove the fill port fitting and insert the funnel tube.

- Fill the pump with 3 quarts of oil, dipping a finger into the oil fill hole throughout the process to check the level.

- Once the oil covers your full fingernail, the pump is filled to the desired level, just under the oil fill port opening.

- Reinstall the fill port fitting.

- Reinstall the red plastic breather cap, and do not over-tighten.

- Repeat steps 4 through 12 for the second pump.

- After the oil has been changed, double-check all drain plugs, fittings, and caps to ensure they are tight.

- Remove the lockout tags from HPP1 and HPP2 disconnects in the MCC.

- Test both HPP1 and HPP2 functions manually with iPad

- Turn HPP1 and HPP2 functions back to Auto on iPad

- Recycle old oil and clean any spills.

Instructional Video:

High-Pressure Pump Oil Change on Vimeo