### Work Instructions: Cleaning a Reclaim Basket strainer

Preliminary Note:

- This should be completed every morning before opening.

Procedure:

1. Place reclaim into “Hand Mode”

2. Loosen dog bone ears on the reclaim basket and take off the cover.

3. Pull out the strainer basket.

4. Spray off mud and other debris from the basket.

5. Place the strainer basket back inside.

6. Put the cover and dog ear bones back on, but do not tighten them.

7. Put reclaim back into “Auto” and press “Prime now”

8. Wait for the water to build and start overflowing from the basket to release all of the air.

9. When water starts to overflow tighten the dog bone ears back down.

10. Reclaim should go back into “Auto” mode after the cover on the basket is tightened down.

Instructional Video:

### Reclaim Maintenance on Vimeo