Work Instructions: Replacing the Center Sleeve Mitter clothes

Parts Needed:

- Stainless Steel for Mitter Basket with Bushing (P-M2047) https://shop.tommycarwash.com/Sleeve-for-mitter-basket-with-UHMW-bushing-1-ID

- Disk (P-M2020) P-M2020|Tommy Car Wash

- Adjustable Wrench

- Ladder

Procedure:

- Press a Wash stop in.

- Loosen the nuts on the set screws, then the set screws themselves.

- lower the sleeve down and remove it from the Mitter camshaft.

- Remove the Mitter camshaft from the Mitter basket shaft.

- Install the new center sleeves on to Mitter basket shaft.

- Move the basket to line up the new Mitter sleeve with the Mitter camshaft and slide the sleeve on to camshaft.

- Once the sleeve is on the Mitter camshaft at the height it was originally at, tighten the set screws in and then tighten the nuts on the sleeve.

- Pull the wash stop out and test the Mitters for correct operation.

Instructional Video:

Center Sleeve Mitter on Vimeo