### Work Instructions: How to replace the Ball and disk on the Mitter Rods

Parts Needed:

- (4) Ball & Disk Sets (P-M2003): P-M2003|Tommy Car Wash

Tools Needed:

- Vise Grips

- 1/2 Impact with 3/4" Deep Well Socket

- Ratchet Strap

- Ladder

Procedure:

- Engage a Wash Stop.

- Position the ladder on the driver’s side of the Mitter Basket.

- Place a ratchet strap around the frame of the Mitter basket up to the framework of the All-In-One Combo Unit.

- Tighten the ratchet strap, lifting the Mitter basket slightly.

- With a vise grip, hold Ball & Disk shaft.

- While holding the shaft, remove the bottom nut with an Impact and Socket Wrench.

- Be careful not to drop the washer, Ball, and Disk from the bottom of the shaft.

- Remove the shaft from the top of the Combo Unit.

- Inspect the shaft on both ends for wear on thickness.

- Install the upper Ball & Disk.

- Reinstall the shaft.

- Install new bottom Ball & Disk and washer.

- Hold the shaft with Vise Grip and tighten the nut back into its original location.

- Repeat for the remaining three Ball & Disk drive shafts.

- Disengage the Wash Stop.

- Turn on the hydraulics.

- Watch and listen for level motion.

Instructional Video:

Ball and Disk Replacement on Mitter on Vimeo