Work Instructions: How to Replace Rocker Brush Cloth

Preliminary Note:

- Inspect the cloth every 100,000 cars. Replace if the cloth has excess wear.

Be sure to order rocker brush cloth for site-specific equipment.

- If the site has electric motors, order a rocker brush for electric motors.

- If the site has Hydraulic motors, order a rocker brush for hydraulic motors.

Parts Needed:

- Rocker Brush Cloth Hydraulic (P-M2187): P-M2187|Tommy Car Wash

- Rocker Brush Cloth Electric (E-M-2020-1-R)

Tools Needed:

- Impact gun

- 12” extension bar for impact gun

- 3/4” socket

- 3/8” wrench

- Adjustable wrench

Procedure:

  - Press in a Wash stop.

  - Turn equipment outputs to “off” on the Tommy Controller.

  - Use a 3/8” wrench or adjustable wrench to remove the screws from the rocker brush cover. Keep the screws in a safe place.

  - Once the cover is removed, use an impact gun with a ¾” socket to remove the 4 nuts from the mounting bolts. (Hydraulic sites will need a 12” socket extension to reach the nuts.)

  - Once the nuts are removed, place all the hardware, flat washer, lock washer, and nuts in a safe place to re-use.

  - Lift and remove the old rocker brush from the mounting plate.

  - Place the new rocker brush onto the mounting plate.

  - Put the hardware back onto the mounting bolts and tighten down the nuts.

  - Once the nuts have been tightened down, put the cover on to new rocker and tighten the screws down.

  - Repeat through the rest of the rockers if replacing all at the same time.

  - Pull out the wash stop

  - Turn on the rockers and watch the rockers spin. Check for no issues.

  - Turn equipment outputs back to “Auto” on the Tommy Controller.

Instructional Video:

Replace Rocker Brush on Vimeo