Work Instructions: How to Replace Solenoids on 2020 Smart Pods

Preliminary Note: These Instructions are applicable for 2020 Smart Pods only.

Parts Needed:

- Solenoid (P-AIR-2244) https://shop.tommycarwash.com/P-AIR-2244?page=1

Tools Needed:

- Small Phillips Head Screwdriver

Procedure:

- Press a wash stop button in.

- Turn off the air pressure to the detergent pod by closing the ball valve.

- Using a small Phillips head screwdriver, loosen and remove the screws holding the solenoid in place. Keep screws in a safe place.

- After the screws are removed, remove the solenoid.

- Place the new solenoid into place and tighten the screws back into place.

- After the new solenoid is mounted, turn the air pressure back on to the detergent pod by opening the ball valve.

- Pull out the wash stop.

- Test the function of the solenoid that has just been replaced.

- If the function does not turn on, check what may have been missed, (a wash stop pressed in still, air pressure still off, etc.)

- If the function works, make sure to turn the function back to “Auto” on Tommy Controller.

Instructional Video:

Replace Bad Coil on Solenoid in Detergent Pod on Vimeo