### Work Instructions: How to clean out the Foam Generators

Parts Needed:

- Foam Generator (P-CHE-2132): https://shop.tommycarwash.com/Twist-n-Kleen-Foam-Generator-by-Mr-Foamer

- Foam Generator Insert (P-CHE-2140): https://shop.tommycarwash.com/Insert-for-Twist-and-Clean-Foam-Generator

Procedure:

- Disconnect the air line and detergent line from the foam generator.

- Unscrew the foam generator from the Nozzle fitting.

- Remove the cap of the Foam generator and remove the foam insert from the foam generator.

- Rinse and clean any residue that is on the insert. Foam inserts with excess build-up can be soaked overnight in a 24:1 dilution ratio of Tommy Brite to water for extra cleaning.

- Rinse and clean out the inside of the foam generator shell.

- Reinsert the clean foam insert back into the clean foam generator shell and screw the cap back on.

- Reinstall the newly cleaned foam generator back into its original position.

- Reconnect the airline and detergent line to the Foam Generator.

Instructional Video:

### Foam Generator Clean Out on Vimeo