Work Instructions: Replacing Bump Stops

Parts Needed:

- Bump Stop (P-M1014) https://shop.tommycarwash.com/P-M1014

Tools Needed:

- Channel Locks

Procedure:

- Place the desired wrap assists in “Manual” on Tommy Controller and push the wrap out. This will help give you access to remove the bump stop.

- Twist off the rubber bump stop. You should be able to use just your hands. If not use a medium to large-sized pair of channel locks to twist off.

- Twist on the new rubber bump stop.

- Turn the wrap assists back to “Auto” on the Tommy Controller.

- Push wraps all the way out and release. Watch to make sure the bump stop engages correctly.

Instructional Video:

Replace Bump Stop on Vimeo