### Work Instructions: How to Inspect, Clean out, and Replace Wash Nozzles, including High-Pressure Nozzles

Parts Required:

- Gray 10E80 Nozzle (P-CHE-2108)

- Red 03E80 Nozzle (P-WAT-2135)

- Gray 10E40 Nozzle (P-CHE-2107)

- Yellow 05E80 Nozzle (P-WAT-2133)

- Stainless Steel High-Pressure Rinse Nozzles (P-WAT-2192)

- Stainless Steel K-Nozzles (P-CHE-2049)

- Stainless Steel Tire Shine Nozzles (P-WAT-2243)

- Spinning High-Pressure Blaster Nozzles (P-CHE-2138)

Tools Required:

- Safety glasses and gloves

- 9/16" Wrench

- Adjustable wrench

- Pliers

- Thread tape

- Welding top Cleaner

Procedure:

- Ensure the wash is not running.

- Put on gloves and safety glasses.

- Remove the clogged nozzle from the arch.

- Rinse the nozzle with water and/or remove debris with a small welding tip cleaner. Do not blow on nozzles.

- Place Thread tape on the nozzle’s threads

- Reinstall the cleaned nozzle and referenced the Nozzle Standard Guide for location.

- Test the function and inspect for leaks.

Triple Threat High-Pressure Nozzles:

- Ensure the wash is not running.

- Remove the nozzle from the high-pressure blaster manifold.

- Place the nozzle halfway in the vice, and use pliers to remove the stainless-steel cap.

- Using 2 adjustable pliers/wrenches, loosen the thin retaining nut on the back of the nozzle.

- Remove the other half of the stainless-steel cap.

- Split the inner body of the nozzle to reveal the internal components, taking note of how they are put together.

- Clean/rinse each component individually.

- Reassemble the nozzle in reverse order.

- Place Thread tape on the nozzle fitting

- Reinstall the nozzle in the high-pressure blaster manifold.

- Test the function and inspect for leaks.

Helpful Documents:

Tommy Tunnel Nozzle Standard Guide

Instructional Video:

Inspect and Clean Nozzles in the Wash Draft 1 on Vimeo