Work Instructions: How to clean out the Body Wax Manifold, Rain Bar, and Foamers

Potential Parts Needed:

- C-Sprayer (P-M2086)

- Foam Generator (P-CHE-2132)

- Foam Generator Insert (P-CHE-2140)

Tools Needed:

- Tommy Brite or Dawn Dish Soap

- Power Sprayer

- Scotch-Brite Pad

- Ladder

Procedure:

- Remove all three foamers from the manifold.

- Take out the foam inserts and clean or replace them depending on the wear/age of the inserts.

- Clean out the foam generators with Scotch-Brite pad, and Dawn Dish Soap, and rinse with the bay wash gun.

- Remove the black foaming sprayers.

- Spray out both sprayers with the bay wash gun.

- Use the bay wash gun and spray out the wax manifold starting at one end. Repeat from the other end.

- Spray each hole from underneath the manifold using the bash wash gun.

- Reinstall all foamers and spray nozzles on the Body wax manifold.

- Manually fire the Body Wax function to check correct coverage.

- If you have holes on the manifold bar that are still clogged, you will need to reclean the Body Wax manifold bar.

Instructional Video:

Body Wax Rain Bar Manifold on Vimeo